# Annex 1 to the Sales Agreement – Required technical specifications for the subject of performance

The subject of performance supplied by the Seller complies with the following requirements:

| Visual stimulation system allowing separate 2D imaging on a monitor and 3D imaging on separate binocular glasses | The Seller fills in the trade name of the performance tendered |
| --- | --- |
| Description of the system and the required technical parameters: | Technical parameters of the performance tendered: |
| The supplied system must be compatible with the existing environment –MRI Siemens device, that is, 3T MRI Siemens Skyra and 32- and 64-channel head coil. The contracting authority requires compatibility in order to use the stimulation system being procured in combination with the existing MRI system installed. That means that the stimulation system procured must physically fit in the existing MRI system, and the magnetic fields of the new and the existing device must not interfere with each other. | The Seller describes the parameters of the performance tendered. |
| High-resolution monitor (minimum of 1920 x 1080 pixels) and a minimum 40" diagonal. | The Seller fills in the trade name of the monitor and its technical parameters. |
| 3D full-HD stereoscopic glasses. | The Seller describes the parameters of the performance tendered. |
| Option of setting the interocular distance and dioptric correction. | The Seller describes the parameters of the performance tendered. |
| Option of eye-tracking. | The Seller describes the parameters of the performance tendered. |
| Manual control for the patient with at least 3 functional buttons allowing feedback and response to the visual stimuli. | The Seller describes the parameters of the performance tendered. |
| Synchronisation unit for synchronisation with the control pulse from the MRI system. | The Seller describes the parameters of the performance tendered. |
| Control unit for the room of the MRI technician to control the system supplied. | The Seller describes the parameters of the performance tendered. |
| Sizes of the rooms where the supplied stimulation system will be placed: Room P49 – 5.2x5.7 meters; Control room for the operator and the control system P50 – 2x4 metres. |  |