# Annex No. 1 of the Purchase Contract – Required Technical Specification of the Subject of Performance

| Rodent Ocular Background Imaging System | SELLER COMPLEMENTS THE COMMERCIAL NAME OF THE GOODS OFFERED |
| --- | --- |
| Description of performance and required technical parameters: |  |
| - Optics system designed specifically to image the rodent eye (3mm for mice and 6mm for rats) Mouse- and rat-specific lens sets, delivering 50-degree field-of-view for real-time, in vivo, longitudinal studies of rodents and similar small animals. | Seller states the parameters of the offered device. |
| - Live, full-color display of the current field of view of the camera, with support to capture both video and still images, with integrated tools to select, review, adjust (brightness, contrast, channels, etc.), compare, and export frames from videos and still images. | Seller states the parameters of the offered device. |
| - An excitation filter wheel and a barrier filter wheel, with preinstalled filter sets for color fundus and GFP imaging, with slots for two additional filter sets. | Seller states the parameters of the offered device. |
| - Integrated subject animal information management (ID, species, strain, born-on date, etc.), including support for user assigned data tags at the animal subject, imaging study, and image levels. All captured meta data can be exported to accompany images and videos for further study and analysis, including as inputs to machine learning and AI systems. | Seller states the parameters of the offered device. |
| - Třísenzorový kamerový subsystém s nízkým Low noise 3-sensor camera subsystem, delivering 3 μm resolution for mouse retina and 6 μm resolution for rat retina in both color fundus and fluorescent imaging studies. | Seller states the parameters of the offered device. |
| - Adjustable digital gain and frame rate to tease out the maximal amount of available signal for faint fluorophores. | Seller states the parameters of the offered device. |
| - Imaging system for fluorescein angiography with sufficient resolution to observe red blood cell circulation, and with the ability to focus the camera on the retinal vasculature prior to engaging the GFP filters to allow for high quality imaging of early profusion. | Seller states the parameters of the offered device. |
| - Modulární Modular benchtop design of a unified system that supports multiple imaging modalities in a small physical footprint to optimize lab space and minimize the need to move subject animals between devices, including color fundus and fluorescent imaging, optional image-guided OCT, image-guided laser, optional image-guided focal ERG, and optional slit lamp components for extended studies. | Seller states the parameters of the offered device. |
| - A camera system that includes a micro-adjustable animal stage with platens designed specifically for mice or rats with 2 degrees of rotation and 3 degrees of translation. | Seller states the parameters of the offered device. |
| - Support to create individual usernames / passwords to control access to the system and data, and the ability to assign users to separate labs, allowing the system to be shared between labs while providing segregation of research data. | Seller states the parameters of the offered device. |
| - Mouse lens kit | Seller states the parameters of the offered device. |